Day 4: Gate-Level Simulation (GLS), Blocking vs. Non-Blocking in Verilog, and Synthesis-Simulation Mismatch

- Gate-Level Simulation (GLS)

- Blocking vs. Non-Blocking Assignments in Verilog

- Synthesis-Simulation Mismatch

Table of Contents

- 1. Gate-Level Simulation (GLS)

- 2. Synthesis-Simulation Mismatch

- 3. Blocking vs. Non-Blocking Assignments in Verilog

  - 3.1 Blocking Statements

  - 3.2 Non-Blocking Statements

  - 3.3 Comparison Table

- 4. Labs

- 5. Summary

1. Gate-Level Simulation (GLS)

GLS stands for Gate-Level Simulation. It is a critical verification step in the VLSI design flow where the synthesized gate-level netlist of a digital circuit is simulated to validate:

- Functional correctness

- Timing behavior

- Power estimates

- Test structures (e.g., scan chains for DFT)

Why Perform GLS?

- Synthesis Validation: Ensures that synthesis tools faithfully translate RTL into gates.

- Timing Verification: Simulates with realistic delays (from SDF files), allowing you to check for timing violations (e.g., setup/hold errors).

- Testability: Confirms that scan chains and other test features work post-synthesis.

When is GLS Performed?

- After synthesis: Once the RTL is converted into a gate-level netlist.

- Before physical design: To catch issues early, before layout.

Types of GLS

- Functional GLS: Logic-only simulation, often with zero or unit delays.

- Timing GLS: Uses annotated timing data to check real-world timing behavior.

2. Synthesis-Simulation Mismatch

A synthesis-simulation mismatch occurs when the simulation results of RTL (pre-synthesis) do not match simulation results of the gate-level netlist (post-synthesis) or hardware. Reasons include:

- Non-synthesizable constructs: Use of delays, initial blocks, or other code not supported by synthesis.

- Incomplete or ambiguous coding: E.g., missing else clauses, improper sensitivity lists.

- Tool interpretation differences: Simulation and synthesis tools may interpret ambiguous RTL differently.

Key Point: Always write synthesizable, unambiguous RTL and follow good coding practices to minimize mismatches.

3. Blocking vs. Non-Blocking Assignments in Verilog

Verilog offers two types of procedural assignments:

3.1 Blocking Statements (=)

- Syntax: =

- Execution: Sequential, executes immediately.

- Suitable for: Combinational logic (e.g., always @(*)).

- Example:

always @(*) y = a & b;

3.2 Non-Blocking Statements (<=)

- Syntax: <=

- Execution: Scheduled, executes concurrently at the end of the time step.

- Suitable for: Sequential logic (e.g., always @(posedge clk)).

- Example:

always @(posedge clk) q <= d;

3.3 Comparison Table

| Blocking (=) | Non-Blocking (<=) |
| --- | --- |
| Uses = operator | Uses <= operator |
| Sequential, immediate execution | Concurrent, scheduled at end of timestep |
| Updates happen instantly in code order | Updates applied after time step |
| For combinational logic, temp variables | For sequential logic, registers/flip-flops |
| Infers combinational logic (gates) | Infers sequential logic (flip-flops) |

4. Labs

Lab 1: Ternary Operator MUX

Verilog code for a simple 2:1 multiplexer using a ternary operator:

module ternary_operator_mux (input i0, input i1, input sel, output y);

assign y = sel ? i1 : i0;

endmodule

- Function: y = i1 if sel = 1; else y = i0.

Lab 2: Synthesis Using Yosys

Synthesize the above MUX using Yosys.
Follow the standard Yosys synthesis flow.

Lab 3: Gate-Level Simulation (GLS) of MUX

Run GLS for the synthesized MUX.
Use this command (adjust paths as needed):

iverilog /path/to/primitives.v /path/to/sky130_fd_sc_hd.v ternary_operator_mux.v testbench.v

Lab 4: Bad MUX Example (Common Pitfalls)

Verilog code with intentional issues:

module bad_mux (input i0, input i1, input sel, output reg y);

always @ (sel) begin

if (sel)

y <= i1;

else

y <= i0;

end

endmodule

Issues:

- Incomplete sensitivity list: Should include i0, i1, and sel.

- Non-blocking assignment in combinational logic: Should use blocking assignments (=).

Corrected version:

always @ (*) begin

if (sel)

y = i1;

else

y = i0;

end

Lab 5: GLS of Bad MUX

Perform GLS on the bad_mux.
Expect simulation mismatches or warnings due to above issues.

Lab 6: Blocking Assignment Caveat

Verilog code:

module blocking_caveat (input a, input b, input c, output reg d);

reg x;

always @ (*) begin

d = x & c;

x = a | b;

end

endmodule

What’s wrong?

- The order of assignments causes d to use the old value of x—not the newly computed value.

- Best Practice: Assign intermediate variables before using them.

Corrected order:

always @ (*) begin

x = a | b;

d = x & c;

end

Lab 7: Synthesis of the Blocking Caveat Module

Synthesize the corrected version of the module and observe the results.

5. Summary

- Gate-Level Simulation (GLS): Validates netlist functionality, timing, and testability after synthesis.

- Synthesis-Simulation Mismatch: Avoid by using synthesizable, unambiguous RTL code.

- Blocking vs. Non-Blocking: Use blocking (=) for combinational, non-blocking (<=) for sequential logic.

- Labs: Reinforce key concepts and highlight common RTL pitfalls.

Day 5: Optimization in Synthesis

Contents

- 1. If-Else Statements in Verilog

- 2. Inferred Latches in Verilog

- 3. Labs for If-Else and Case Statements

  - Lab 1: Incomplete If Statement

  - Lab 2: Synthesis Result of Lab 1

  - Lab 3: Nested If-Else

  - Lab 4: Synthesis Result of Lab 3

  - Lab 5: Complete Case Statement

  - Lab 6: Synthesis Result of Lab 5

  - Lab 7: Incomplete Case Handling

  - Lab 8: Partial Assignments in Case

- 4. For Loops in Verilog

- 5. Generate Blocks in Verilog

- 6. What is an RCA (Ripple Carry Adder)?

- 7. Labs on Loops and Generate Blocks

  - Lab 9: 4-to-1 MUX Using For Loop

  - Lab 10: 8-to-1 Demux Using Case

  - Lab 11: 8-to-1 Demux Using For Loop

  - Lab 12: 8-bit Ripple Carry Adder with Generate Block

- Summary

1. If-Else Statements in Verilog

If-else statements are used for conditional execution in behavioral modeling, typically within procedural blocks (always, initial, tasks, or functions).

Syntax

if (condition) begin

// Code block executed if condition is true

end else begin

// Code block executed if condition is false

end

- condition: An expression evaluating to true (non-zero) or false (zero).

- begin ... end: Used to group multiple statements. Omit if only one statement is present.

- The else part is optional.

Nested If-Else

if (condition1) begin

// Code for condition1 true

end else if (condition2) begin

// Code for condition2 true

end else begin

// Code if no conditions are true

end

2. Inferred Latches in Verilog

Inferred latches occur when a combinational logic block does not assign a value to a variable in all possible execution paths. This causes the synthesis tool to infer a latch, which may not be the designer’s intention.

Example of Latch Inference

module ex (

input wire a, b, sel,

output reg y

);

always @(a, b, sel) begin

if (sel == 1'b1)

y = a; // No 'else' - y is not assigned when sel == 0

end

endmodule

Problem: When sel is 0, y is not assigned, so a latch is inferred.

Solution: Add Else or Default Case

module ex (

input wire a, b, sel,

output reg y

);

always @(a, b, sel) begin

case(sel)

1'b1 : y = a;

default : y = 1'b0; // Default assignment

endcase

end

endmodule

3. Labs for If-Else and Case Statements

Lab 1: Incomplete If Statement

module incomp_if (input i0, input i1, input i2, output reg y);

always @(*) begin

if (i0)

y <= i1;

end

endmodule

Lab 2: Synthesis Result of Lab 1

Lab 3: Nested If-Else

module incomp_if2 (input i0, input i1, input i2, input i3, output reg y);

always @(*) begin

if (i0)

y <= i1;

else if (i2)

y <= i3;

end

endmodule

Lab 4: Synthesis Result of Lab 3

Lab 5: Complete Case Statement

module comp_case (input i0, input i1, input i2, input [1:0] sel, output reg y);

always @(*) begin

case(sel)

2'b00 : y = i0;

2'b01 : y = i1;

default : y = i2;

endcase

end

endmodule

Lab 6: Synthesis Result of Lab 5

Lab 7: Incomplete Case Handling

module bad_case (

input i0, input i1, input i2, input i3,

input [1:0] sel,

output reg y

);

always @(*) begin

case(sel)

2'b00: y = i0;

2'b01: y = i1;

2'b10: y = i2;

2'b1?: y = i3; // '?' is a wildcard; be careful with incomplete cases!

endcase

end

endmodule

Lab 8: Partial Assignments in Case

module partial_case_assign (

input i0, input i1, input i2,

input [1:0] sel,

output reg y, output reg x

);

always @(*) begin

case(sel)

2'b00: begin

y = i0;

x = i2;

end

2'b01: y = i1;

default: begin

x = i1;

y = i2;

end

endcase

end

endmodule

Note: Steps to perform the above labs are shown in Day 1.

4. For Loops in Verilog

A for loop is used within procedural blocks (initial, always, tasks/functions) to execute statements multiple times based on a loop counter.

Syntax

for (initialization; condition; increment) begin

// Statements to execute

end

- Must be inside procedural blocks.

- Synthesizable only if the number of iterations is fixed at compile time.

Example: 4-to-1 MUX Using a For Loop

module mux_4to1_for_loop (

input wire [3:0] data, // 4 input lines

input wire [1:0] sel,  // 2-bit select

output reg y           // Output

);

integer i;

always @(data, sel) begin

y = 1'b0; // Default output

for (i = 0; i < 4; i = i + 1) begin

if (i == sel)

y = data[i];

end

end

endmodule

5. Generate Blocks in Verilog

A generate block is used to create hardware structures such as module instances or logic at compile time. Typically used with for loops and the genvar keyword.

Example

genvar i;

generate

for (i = 0; i < 4; i = i + 1) begin : gen_loop

and_gate and_inst (.a(in[i]), .b(in[i+1]), .y(out[i]));

end

endgenerate

6. What is an RCA (Ripple Carry Adder)?

An RCA adds binary numbers using a chain of full adders. To add n bits, you need n full adders. Each carry-out connects to the carry-in of the next stage.

7. Labs on Loops and Generate Blocks

Lab 9: 4-to-1 MUX Using For Loop

module mux_generate (

input i0, input i1, input i2, input i3,

input [1:0] sel,

output reg y

);

wire [3:0] i_int;

assign i_int = {i3, i2, i1, i0};

integer k;

always @(*) begin

for (k = 0; k < 4; k = k + 1) begin

if (k == sel)

y = i_int[k];

end

end

endmodule

Lab 10: 8-to-1 Demux Using Case

module demux_case (

output o0, output o1, output o2, output o3,

output o4, output o5, output o6, output o7,

input [2:0] sel,

input i

);

reg [7:0] y_int;

assign {o7, o6, o5, o4, o3, o2, o1, o0} = y_int;

always @(*) begin

y_int = 8'b0;

case(sel)

3'b000 : y_int[0] = i;

3'b001 : y_int[1] = i;

3'b010 : y_int[2] = i;

3'b011 : y_int[3] = i;

3'b100 : y_int[4] = i;

3'b101 : y_int[5] = i;

3'b110 : y_int[6] = i;

3'b111 : y_int[7] = i;

endcase

end

endmodule

Lab 11: 8-to-1 Demux Using For Loop

module demux_generate (

output o0, output o1, output o2, output o3,

output o4, output o5, output o6, output o7,

input [2:0] sel,

input i

);

reg [7:0] y_int;

assign {o7, o6, o5, o4, o3, o2, o1, o0} = y_int;

integer k;

always @(*) begin

y_int = 8'b0;

for (k = 0; k < 8; k = k + 1) begin

if (k == sel)

y_int[k] = i;

end

end

endmodule

Lab 12: 8-bit Ripple Carry Adder with Generate Block

module rca (

input [7:0] num1,

input [7:0] num2,

output [8:0] sum

);

wire [7:0] int_sum;

wire [7:0] int_co;

genvar i;

generate

for (i = 1; i < 8; i = i + 1) begin

fa u_fa_1 (.a(num1[i]), .b(num2[i]), .c(int_co[i-1]), .co(int_co[i]), .sum(int_sum[i]));

end

endgenerate

fa u_fa_0 (.a(num1[0]), .b(num2[0]), .c(1'b0), .co(int_co[0]), .sum(int_sum[0]));

assign sum[7:0] = int_sum;

assign sum[8] = int_co[7];

endmodule

Full Adder Module:

module fa (input a, input b, input c, output co, output sum);

assign {co, sum} = a + b + c;

endmodule

Summary

- Use complete if-else and case statements to avoid unintended latch inference.

- For loops and generate blocks are powerful for writing scalable, synthesizable code.

- Always ensure every signal is assigned in every possible execution path for combinational logic.

- Use labs to reinforce concepts with practical Verilog code and synthesis results.