Day 3: Combinational and Sequential Optimization

Welcome to Day 3!
So far, we’ve learned how to simulate and synthesize RTL designs. optimization — techniques that help make circuits smaller, faster, and more power-efficient. Both combinational and sequential designs can be optimized, and we’ll explore some common methods along with hands-on labs.

Agenda

- Constant Propagation

- State Optimization in FSMs

- Cloning for Load Balancing

- Retiming of Registers

- Lab Experiments (6 mini-exercises)

1. Constant Propagation

This technique detects constant values in the design and pushes them forward through the circuit.

How it works:

- The tool scans the code for signals that always hold fixed values.

- Those signals are replaced with constants.

- As a result, large portions of logic can get simplified or even removed.

Advantages:

- Less logic → fewer gates → smaller area

- Shorter paths → improved speed

- Reduced power and resource usage

2. State Optimization

Finite State Machines (FSMs) often carry redundant states or inefficient encodings. State optimization tackles this.

Main steps:

- State Reduction: Merge states that behave identically.

- Encoding: Use efficient state codes (binary, Gray, one-hot, etc.).

- Logic Minimization: Apply Boolean simplification.

- Low-Power Design: Techniques like clock gating reduce unnecessary switching.

End result: more compact FSMs that consume less power and silicon.

3. Cloning

Sometimes a single gate or module drives too many loads, creating long delays. Cloning fixes this by duplicating that logic and distributing the fanout.

Process:

- Identify critical nets with high delay.

- Duplicate (clone) the logic cell.

- Split loads between original and clone.

- Verify timing and power improvements after placement and routing.

Effect: Shorter wires, lower delay, and balanced load distribution.

4. Retiming

Retiming is a powerful optimization for sequential circuits. It involves moving registers (flip-flops) across logic without changing functionality.

Steps:

- Represent the circuit as a graph.

- Shift registers forward or backward to balance delays.

- Ensure setup/hold timing constraints remain satisfied.

- Achieve reduced clock period (higher performance) or lower power.

Think of it as "shuffling registers around" to smoothen performance bottlenecks.

5. Lab Experiments on Optimization

Lab 1: Constant Propagation with Multiplexer

module opt_check (input a , input b , output y);

assign y = a ? b : 0;

endmodule

- If a=1, y=b.

- If a=0, y=0.

- The optimizer replaces this with simpler logic.

Run synthesis and before abc -liberty, insert:

opt_clean -purge

Lab 2: Multiplexer Simplification

module opt_check2 (input a , input b , output y);

assign y = a ? 1 : b;

endmodule

- Works like a mux:

  - y=1 when a=1.

  - y=b when a=0.

Lab 3: Another 2-to-1 Multiplexer Form

module opt_check3 (input a , input b , output y);

assign y = a ? 1 : b;

endmodule

- Equivalent to Lab 2.

- Simplifies to a mux that chooses between constant 1 and b.

Lab 4: Nested Ternary Reduction

module opt_check4 (input a , input b , input c , output y);

assign y = a ? (b ? (a & c) : c) : (!c);

endmodule

- Step-by-step simplification shows this reduces to:

y = a ? c : !c;

Lab 5: D Flip-Flop with Constant Data

module dff_const1(input clk, input reset, output reg q);

always @(posedge clk, posedge reset)

begin

if(reset)

q <= 1'b0;

else

q <= 1'b1;

end

endmodule

- Reset clears q=0.

- Otherwise, the flop always loads 1.

Lab 6: D Flip-Flop Always High

module dff_const2(input clk, input reset, output reg q);

always @(posedge clk, posedge reset)

begin

if(reset)

q <= 1'b1;

else

q <= 1'b1;

end

endmodule

- No matter what, q is always 1.

- Synthesizer will likely optimize this away to a constant net.

Recap

- Constant Propagation → replace constants for simpler circuits.

- State Optimization → reduce redundant FSM states and logic.

- Cloning → duplicate logic to balance loads.

- Retiming → reposition registers to improve timing.

- Labs → saw how optimization changes real Verilog code.